# KeeUnloq

Open-source tools for exploring Microchip KeeLoq keyless remote systems (the old ones).

| KUL FULL v1 PCB rev 0 | KUL LITE v1 PCB rev 0 |
| --- | --- |

## Features

Device comes in two PCB variants: KUL FULL and KUL LITE.

Note: version KUL FULL contains RF receiver and transmitter while KUL LITE does not.

- Receiver module with optocouplers (only with PCB „KUL FULL“)

- Upgrading third-party insecure systems that use fixed-code HCS101 with rolling-code HCS versions (HCS200, HCS300, ...)

- Emulating HCS transmitter (only with PCB „KUL FULL“ or with external RF ASK transmitter)

- KeeLoq HCS series programmer tool (requires PC software)

- PC software for programming HCS series chips, upload & download of collected transmitter data and more

- Firmware upgrade via USB

- Powered externally or by USB

  - 5V DC or 9-24 AC/DC (only with PCB „KUL FULL“)

  - 5V DC (PCB „KUL LITE“)

- Open source

## Modes of operation

Devices with FW v1 support 4 stand-alone (does not require PC connection) modes of operation:

- Mode 1: Receiver module with memory of up to 1000 remote transmitters

- Mode 2: MITM upgrader for upgrading third-party systems (insecure garage door openers)

- Mode 3: Grab, collect and re-transmit with memory of up to 1000 transmissions

- Mode 4: Transmitter emulator

Mode of operation is changed by holding down button S0 and resetting the device (via reset button or by supplying power). LED A will remain ON and current operating mode will be displayed by blinking the LED B. Selecting different mode of operation can now be accomplished by again pressing the button S0 within 5 seconds. If button S0 is not pressed within 5 seconds, the device will exit mode-change procedure and start with operation in currently selected mode.

### Mode 1: Receiver module

Receiver module is used as a stand-alone receiver which is capable of learning up to 2000 KeeLoq remote transmitters. It has 4 digital optocoupler outputs S0-S3 (only PCB „KUL FULL“) which are usually connected to garage door motor controllers. It is also capable of learning insecure HCS101 chips and rolling-code HCS series that require programming before usage.

Please refer to Table 1 in Programming remote transmitters and encoder ICs.

### Mode 2: MITM upgrader

Man-in-the-middle upgrader is a mode of operation where the goal is to upgrade a fixed-code insecure third-party system with a rolling-code transmitter. Rolling-code HCS series are pin-compatible with HCS101 fixed code encoder so modifying existing insecure transmitters should be an easy task by simply replacing ICs and programming new HCS with KeeUnloq device.

In order for this mode of operation to be successful, a third-party system must be modified as shown in the schematic diagram above.

The data line of the third-party system must be intercepted so that any data received by the RF receiver is passed into the KeeUnloq device, and special data line from the KeeUnloq device passed into the third-party system.

This means that KeeUnloq device must learn:

- Fixed-code HCS101 remote control (only one HCS101) that third-party system already recognizes and works with

- Rolling-code transmitter that will be learned by the KeeUnloq device (as many as required)

After installing the KeeUnloq MITM device into the third-party system, regular programming must take place with one exception: The remote transmitter that has been last learned by the KeeUnloq device will be passed/proxied into the third-party system upon reception of a valid learned rolling-code remote transmitter.

Important note: fixed-code remote transmitters cannot be used in this mode during normal operation of the device because learning the last fixed-code remote transmitter is used to pass into the third-party system, so beware.

Please refer to Table 1 in Programming remote transmitters and encoder ICs.

### Mode 3: Grab, collect & re-transmit

This mode is intended for collecting KeeLoq data from the air in order to evaluate the security of third-party systems for vulnerabilities such as: code re-transmission.

When device is in this state, it will listen, grab and collect information and save to internal EEPROM memory. All data can be downloaded to PC software via USB connection.

Pressing & holding button S0 on device will pause the receiver and start re-transmitting entire memory of collected transmitters. After the entire memory is re-transmitted (or button S0 is pressed), device will go back to normal operation of collecting transmitters.

Pressing & holding button S1 on device will pause the receiver and re-transmit the last grabbed transmitter and finally go back to normal operation of collecting transmitters.

Pressing button S2 on device will indicate how many new remote devices have been collected until device has started/booted.

Pressing button S3 on device will indicate how many devices are collected in entire EEPROM memory.

Number of collected transmitters is reported by blinking the LEDs:

- LED A – hundreds

- LED B – tens

- LED C – ones

Example 1: LED A blinks 2 times, LED B does not blink, LED C blinks 7 times; the resulting number is 207.

Example 2: LED A blinks 11 times, LED B blinks 2 times, LED C does not blink; the resulting number is 1120.

### Mode 4: Transmitter emulator

In this mode it is possible to emulate any of the HCS series encoders. It is required to previously program the “transmitter identity” via USB connection and PC software:

- Type of encoder (HCS*)

- Serial number

- Encryption key (if applicable)

- Counter value

Once device is in this mode, all four buttons are used to transmit the data (S0 – S3).

## Programming remote transmitters and encoder ICs

Programming is done by accessing buttons S0-S3 or via PC software when device is connected via USB.

| Program & learn | To learn rolling-code encoder into the device memory, place programming clips on the HCS IC or place it on the PCB programming slot and hold it down firmly. Hold button S0 to enter programming mode and LED B will turn ON. Within 20 seconds press one of the buttons: - S0 – program&enroll 66 bit encoder (HCS200) - S1 – program&enroll 66 bit encoder (HCS201) - S2 – program&enroll 66 bit encoder (HCS300, HCS301, HCS320) - S3 – program&enroll 67 bit encoder (HCS360, HCS361) Each HCS encoder IC will be programmed with a random serial number, random encryption key, random discrimination value (except HCS360 & HCS361 which mirror the serial number value) and random counter value (but kept in the range of 0 – 255). This means that encoder ICs programmed using KeeUnloq device cannot be later enrolled using “Learn from RF” function in case KeeUnloq memory is later deleted - encoder ICs will need to be re-programmed. If it is required that all encoder ICs share the same encryption key, then use the PC tool to program them individually to the master encryption key of your choice. This master key resides in KeeUnloq memory. The device will remain in this state for 20 seconds after the last transmitter has been programmed and will exit automatically. |
| --- | --- |
| Learn from RF | To learn a remote transmitter into the device memory from RF, hold button S1 to enter programming mode and LED B will turn ON. Within 10 seconds press any button on a remote control and LED B will start blinking. Press again the same button on remote control within 5 seconds. If learning process was successful LED C will blink: - 2 times if remote has been recognized using pre-configured master key as a rolling-code encoder - 5 times if remote has been recognized as a fixed-code encoder or if using the pre-configured master key did not decrypt the data successfully so the encoder received will be treated as fixed-code type If programming was not successful LED A will blink: - 2 times if remote is already programmed into the memory (you can try first deleting it and then learning it again) - 3 times if device is in Mode 2 and there is already HCS101 emulating encoder enrolled - 5 times if remote failed to program because second reception did not match the first reception (you can simply try again) - 10 times if this remote is not supported by current firmware The device will remain in this state for 10 seconds after the last transmitter has been received and will exit automatically. This process will enroll a fixed-code remote transmitter HCS101 any time, or a rolling-code transmitter with the encryption key that has been earlier loaded into the KeeUnloq device via USB and PC configuration tool. This also means that every remote transmitter must have this exact encryption key already programmed into them. It will also enroll a rolling-code transmitter even if the encryption key does not match, however it will be treated as a fixed-code transmitter where only serial number is used just like with HCS101. Important note: When device is in Mode 2 (MITM Upgrader) last learned fixed-code HCS101 will be used to pass/proxy into the third-party system. |
| Remove transmitter from memory (one-by-one) | In order to delete a specific remote transmitter from the memory, press and hold button S2. LED B will start blinking. Press any button on a remote transmitter that needs to be deleted from the memory. LED A will turn ON to indicate the successful reception of the code. When remote transmitter has been deleted from the memory, LED C will blink 2 times to confirm deletion, or LED A will blink 2 times to confirm that remote has not been found in memory. The device will remain in this state for 10 seconds after the last transmitter has been received and will exit automatically. |
| Clear all memory | In order to clear all memory from the device, press and hold button S3 until LED A starts blinking. Now within 5 seconds press and hold button S2 until LEDs B&C turn on. Release all buttons and wait until all LEDs finally turn off – the memory is clear. When device is in: - MODE 1: this will delete only learned/enrolled encoders that can operate the device - MODE 2: this will delete only learned/enrolled MITM emulating encoder - MODE 3: this will delete entire grabbed/log table - MODE 4: this will delete nothing |

Table 1: programming remote transmitters and encoder ICs

## Appendix

Example of installation of MITM Upgrader into a garage door opener that supports HCS101. Before continuing make sure that garage door opener is programmed and controllable by the working remote transmitter that has HCS101 encoder inside.

Step 1: access the garage door openers RF receiver

Step 2: cut the data line and pass it through the KeeUnloq device and power up the garage door opener. Now change KeeUnloq mode into the MITM Upgrader (refer to the instructions in this document on how to change the operating mode)

Step 3: perform the Program & learn procedure of the existing HCS101 remote controller into the KeeUnloq

Step 4: order new remote transmitters with rolling-code HCS encoder IC (such as: HCS200, HCS300, …) or remove existing HCS101 encoder IC from your current remote controller

Step 5: perform the Program & learn procedure of the new HCS encoder IC by using the programming clips or by using the pads on KeeUnloq device (in case you haven’t already soldered new HCS encoder IC onto the remote transmitter)

Step 6: test for proper operation of garage door opener with KeeUnloq device and your new rolling-code transmitter